- Any controller MUST perform the following operations:  ___________________, _______________________, __________________________________ and perform feedback.

- Explain what happens when you select the following keystrokes to any configuration screen?

APPLY		______________________________________________________________

UPDATE	______________________________________________________________

OK		______________________________________________________________

- SCADA stands for:  ___________________________________________________________

- Use your knowledge of the Fisher 107 to answer the following questions.

  - Circle the CPU module on the Fisher 107 and mark it with the letters CPU.

  - Circle the MVS card on this Fisher 107 and clearly mark the card with the letters MVS

.

- In the space below draw a simple circuit that contains a 5Ω, a 5Ω and a 4Ω resistor each wired in parallel to a 12 V battery.  Your circuit should also contain a switch in the open position that will prevent current flow to any of the resistors until it is closed.

- Calculate the resistance of an equivalent single resistor to the parallel set you drew in Question 4 above if the switch is closed.

- During installation of a daisy change of three MVS transducers suggest an acceptable address for the three MVS’s in the series.  _________	__________	____________.

- Is the MVS a Digital or Analog device?  ____________________

- Describe the NEC Class I standard

  - Flammable gases are always present in the air in quantities sufficient to produce explosive or ignitable mixtures

  - Flammable gases may be present in the air in quantities sufficient to produce explosive or ignitable mixtures

  - Flammable gases are not present in the air in quantities sufficient to produce explosive or ignitable mixtures

  - All of the above

- Division 2 means that:

  - Locations are presumed to be hazardous at all times.

  - Locations are presumed to be hazardous during normal operations

  - Locations are presumed to be hazardous under abnormal conditions

  - Locations are presumed to be non-hazardous at all times.

- An MVS (Multi-variable Sensor) is a sensor that simultaneously:

  - Measures temperature, flow and density of the natural gas

  - Measures flow, density and composition of the natural gas

  - Measures static pressure, differential pressure, and temperature of the natural gas

  - Calculates gas flow using the AGA 1985 or 1992 standard.

- A new MVS from the factory will have an address of:

  - 5

  - 10

  - 1

  - 0

- What MUST be true of ALL of these critical network settings between different radios? ______________________________________________________________________________

R1

In the following diagram, with the switch closed, how much current will be carried by each of the resistors in the circuit below if R1 is 10Ω and R2 is 2Ω and R3 is 2Ω.  SHOW your work.

12V

R2

R3

- Examine the screen shots below:  Will this set up screen allow Production Manager to run on your ROC?  Circle the correct answer  	YES	NO.  IF NO explain the problem

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Circle the appropriate menu item to select in order to access the Production Manager Program.

- In the following screen circle the control button or menu item you select in order to define which input Production Manager should look to for the value of the casing pressure.

- Which POINT TYPE in the screen below will contain the input from the casing pressure. Circle the correct choice on the screen shot.

- On the screen shot below, circle the correct Parameter with which to obtain the values of casing pressure in the correct units.

- The following configuration screen has the monitor plunger arrival box checked.

On the following diagram CIRCLE the specific device that will be monitored in order to sense the arrival of the plunger at the lubricator. What is the name of this device ______________________________?

- Place a number next to the following steps in the Plunger Lift Cycle to indicate the correct sequence assuming that the plunger starts at the bottom of the well and the gas flow control valve is initially closed.  Write the same corresponding number onto the screen shot to indicate which operating block will be illuminated during the step in the sequence.  The first one is done for you as an example.  (Note:  The same block may be use for more than one process)  Assume a conventional plunger set up.

_______ Plunger arrival sensor is triggered

_______Open the control valve to initial gas flow

__4___ Afterflow

_______Close the control valve

_______ Plunger is at the bottom of the well

_______Tubing pressure is minimum

- In the spaces next to each radio, identify whether the radio is a repeater, repeater-slave, or slave using the letters R, RS, or S.  This time you MUST correctly distinguish between the use of Repeter or RS – Repeater Slave when you mark the radios.  Connect the wells together with STRAIGHT lines to indicate the transmission paths you have defined.  Correctly define the subnet ID’s for each of the radios in the network (HINT:  Place the number of the repeater inside the little box representing the radio).

- Define the following terms for a PID Controller.

  - P = ______________________ but the ROC uses the inverse called __________.

  - I = _______________________ but the ROC uses the inverse called ________

  - D = ______________________ but the you will almost NEVER use this parameter

- Select the two most important decisions when setting up a PID controller on the ROC

  - Definer the color of the screen to use.

  - Perform a loop check in auto mode

  - Choose a name for the PID Controller.

  - Prior to changing the set up make sure the controller is disabled.

  - Perform a loop check in manual mode

  - Prior to changing the set up make sure the controller is enabled.

- What ONE change MUST be made to this PID Loop in order to stop the rapid oscillations in the Process variable and the Valve Output indicated in the trend line.

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________